Project 5_Phase1

Title  			(0.1 points)

Collision avoidance for a robotic manipulator in a virtual environment

Introduction   	(0.25 point)

- Background	: VxSIM is a virtual environment engine used by my lab at UMBC and contains built-in virtual camera and LiDAR sensors that I can place in a recreated version of the lab containing a digital twin of the Dobot CR3. We can place obstacles/customize this space as we see fit, acting as a viable alternative to Gazebo (at least within the scope of this project). Doing so would also help to further the progress of our research.

- NOTE: This software is proprietary and I would not be able to give the simulation software/environment in the submission for testing/verification on the grader’s end

Goal                            (0.15 points)

- In a virtual simulation, navigate to a goal position in the virtual environment while avoiding obstacles placed in the scene

Method		(1 points)

- Path planning Method: RRT through joint space (6-DOF). Using forward kinematics via DH parameterization (already done), the skeleton of the robot can be uploaded to a matplotlib file (half-done). Using the virtual LiDAR, the obstacles can also be uploaded to the virtual environment. From this, I intend to have RRT run until a series of configurations are found that can be stitched together to navigate the robot from its current configuration to the goal position (orientation convergence is also achievable, but for simplicity I plan to focus on position.

- Software: VxSIM

- Hardware requirements: Laptop to run the simulation

- Title of the paper/papers that you will implement or use:

  - Obstacle Avoidance Path Planning for Robotic Manipulators Based on an Improved Bidirectional RRT* Algorithm

  - Obstacle Avoidance of Robotic Arm in Path Planning Based on RRT Algorithm

Timetable	       	(0.25 points)

- LiDAR Sensor fusion of local virtual environment into MatPlotLib along with robot skeleton (2-3 days)

- Joint-space RRT implementation for 6-DOF robot (3–4 days)

- Forward kinematics-based collision checking for sampled configurations (2–3 days)

- Path stitching and goal condition verification (1–2 days)

- Uploading the found trajectory to be replayed in VxSIM simulation (collision detection active)  (1–2 days)

- Testing and debugging (3–4 days)

- Final report, results, and presentation preparation (2–3 days)

References                (0.25 points)

X. Yan, T. Xiong, D. Xiang, D. Zhao and Z. Li, "Obstacle Avoidance Path Planning for Robotic Manipulators Based on an Improved Bidirectional RRT* Algorithm," 2025 7th International Conference on Robotics, Intelligent Control and Artificial Intelligence (RICAI), Hangzhou, China, 2025, pp. 99-103, doi: 10.1109/RICAI68060.2025.11385154. keywords: {Heuristic algorithms;Trees (botanical);Prediction algorithms;Planning;Trajectory;Collision avoidance;Splines (mathematics);Manipulator dynamics;Standards;Convergence;six-degree-of-freedom robotic manipulators;Obstacle avoidance path planning;Bi-RRT* algorithm;Dynamic biased sampling;Natural spline curve},

B. He, "Obstacle Avoidance of Robotic Arm in Path Planning Based on RRT Algorithm," 2025 6th International Conference on Electrical Technology and Automatic Control (ICETAC), Nanjing, China, 2025, pp. 685-689, doi: 10.1109/ICETAC65964.2025.11144270. keywords: {Vibrations;Service robots;Heuristic algorithms;Blindness;Arms;Manipulators;Path planning;Collision avoidance;MATLAB;Stress;path planning;RRT algorithm;RRT smooth algorithm;obstacle avoidance;robotic arm},